ATSF-SP JOINT LINE

OPERATING RULES

# SECOND EDITION

## EFFECTIVE – July 1, 2002

The rules herein govern the operation of the railroad and must be complied with by all employees regardless of gender whose duties are in any way affected thereby.

They supersede all previous rules and instructions inconsistent therewith.

ADOPTED BY:

LA MESA MODEL RAILROAD CLUB

INDEX

Absent or Improperly Displayed Signals ………          8, 24

Absolute-Permissive Block………………………….      27

Automatic Block Signal System……………………..      25

Block Signal Aspects and Indications……..……… .       22

Block Signal and Interlocking  Rules……………….       22

Block System Rules…………………………………       25

Centralized Traffic Control (CTC) Rules.…………..       27

Color Signals………………………………………..         6

Definitions…………………………………………..         3

Engine Bell………………………………………….         8

Engine Whistle Signals……………………………..          7

Fixed Signals……………………………………….        22

Forms of Train Orders……………………………..         14

General Notice……………………………………..           3

General Regulations………………………………..        28

General Rules………………………………………          3

Hand, Flag and Lamp Signals……………………...          7

Headlights…………………………………………...        7

Interlocking……………………..…………………..       26

Letter-Type Indicators………………………………       26

Main Track Authorization………………………….          8

Movement of Trains and Engines………………….          8

Radio Rules………………………………………. .        30

Rules for Movement by Train Orders……………..         11

Rules for Single and Double Track………………..           5

Signals and Their Use……………………………. .          6

Spring Switches……………………………………        25

Standard Time……………………………………...          5

Superiority of Trains……………………………….          8

Timetables………………………………………….         5

Track and Time Limits…………………………….        27

Yard Limits………………………………………..          5

GENERAL NOTICE

SAFETY is of the first importance in the discharge of duty.  The term employee applies to anyone operating the ATSF-SP JOINT LINE.  Obedience to the rules is required.  To enter or remain in service is an assurance of willingness to obey the rules.  To obtain promotion, ability must be shown for greater responsibility.

GENERAL RULES

A. Employees whose duties are prescribed by these rules must be provided with a copy which they must have available while on duty.  Employees whose duties are in any way affected by the timetable must have a copy of the current timetable and supplements, if any, with them while on duty.

B. Employees must be familiar with and obey all rules and instructions.  If in doubt as to their meaning, they must apply to the proper authority for an explanation.

C. Employees must pass the required examination(s).

E. Employees must render every assistance in their power in carrying out the rules and instructions and must report promptly to the proper official any violation thereof.

G. The use of alcoholic beverages, intoxicants or narcotics by employees subject to duty is prohibited.

H. The use of tobacco by employees while on duty in the presence of the public is prohibited.

J. Employees reporting for duty must be neat and clean in appearance, suitably clothed and wear their hair in a manner to permit safe performance of their duties.

M. Carelessness by employees will not be condoned and they must exercise care to avoid injury to themselves, others and the railroad.

DEFINITIONS

ABSOLUTE-PERMISSIVE BLOCK (A-PB).   A block system within which the movement of trains on a designated section of track or tracks is directed by signals automatically controlled and without train-order authority and without the superiority of trains.

ABSOLUTE SIGNAL.  A home signal, the indications of which authorize and govern the movement of trains and engines and supersede the superiority of trains.

APPROACH SIGNAL.  A fixed signal used in connection with a home signal to govern the approach thereto.  (For example, see Rule 285).

AUTOMATIC BLOCK SYSTEM (ABS).  A series of consecutive blocks governed by block signals actuated by a train or certain conditions affecting the use of a block.

BLOCK.  A length of track between consecutive block signals governing in one direction, or from a block signal to a defined limit.

BLOCK SIGNAL.  A fixed signal at the entrance of a block, or within a block, to govern trains entering and using that block.

BLOCK SYSTEM.  A series of consecutive blocks within A-PB, ABS, CTC and interlockings.

CENTRALIZED TRAFFIC CONTROL (CTC).  A method of operation by means of which the movement of traffic over routes and through blocks on a designated section of track or tracks is directed by signals controlled from a designated point without requiring use of train orders and without regard to superiority of trains.

CONTROLLED SIDING.  A siding within CTC or interlocking limits by which trains enter or leave by signal indication.

CROSSOVER.  A track connection between two adjacent tracks.

CURRENT OF TRAFFIC.  Movement of trains on a main track, in one direction, specified by the rules.

DIVISION.  That portion of a railroad assigned to the supervision of a superintendent.

DOUBLE TRACK.  Two main tracks, on one of which the current of traffic is in a specified direction, and on the other in the opposite direction.

DUAL CONTROL SWITCH.  A power-operated switch, also equipped for hand throw operation.

ENGINE.  A unit propelled by any form of energy, or combination of such units operated from a single control, used in train or yard service.

EXTRA TRAIN.  A train not authorized by a timetable schedule.  It may be designated:

EXTRA, for any extra train except work extra; or

WORK EXTRA, for work train extra.

FIXED SIGNAL.  A signal of fixed location indicating a condition affecting the movement of a train, such as train-order, interlocking, absolute or automatic block signals; stop signs; yard limit signs or speed signs.  Automatic block signals will have a number plate attached to their mast.

GRADE SIGNAL.  An automatic block signal with disk on mast bearing letter “G”.

HOME SIGNAL.  A fixed signal at the entrance of a route or block to govern trains entering and using that route or block.

INTERLOCKING.  An arrangement of signal appliances so interconnected that their movements must succeed each other in a predetermined order.  It may be operated manually or automatically.

INTERLOCKING LIMITS.  The tracks between opposing home signals of an interlocking.

INTERLOCKING SIGNAL.  A home signal of an interlocking.

MAIN TRACK.  A track extending through yards and between stations, upon which trains are operated by timetable or train order, or both, or the use of which is governed by signal indication.

MARKER.  A red light or other prescribed signal affixed to rear end of equipment being operated as a train.

PILOT.  Employee assigned to a train when the engineer or conductor is not acquainted with the rules or portion of railroad over which the train is to be moved.

REGULAR TRAIN.  A train authorized by a timetable schedule.

SCHEDULE.  That part of a timetable which prescribes class, direction, number and movement for a regular train.

SECTION.  One of two or more trains running on the same schedule displaying signals or for which signals are displayed.

SIDING.  A track auxiliary to the main track for meeting or passing trains.

SIGNAL ASPECT.  Appearance of a fixed signal conveying an indication as viewed from the direction of an approaching train.

SIGNAL INDICATION.  Information conveyed by the aspect of a fixed signal.

SINGLE TRACK.  A main track on which trains are operated in both directions.

SPEEDS.

MEDIUM SPEED: A speed not exceeding forty (40) miles per hour.

RESTRICTED SPEED:  Proceed prepared to stop short of train, obstruction or switch not properly lined and look out for broken rail, not exceeding twenty (20) miles per hour.

WITH CAUTION: To run at reduced speed, according to conditions, prepared to stop short of a train, engine, car, misplaced switch, derail or other obstruction, or before reaching a stop signal.

SPRING SWITCH.  A switch equipped with a spring mechanism to restore the switch points to original position after having been trailed through.

STATION.  A place identified in the timetable by name.

SUBDIVISION.  A portion of a division designated by timetable.

SUPERIOR TRAIN.  A train having precedence over another train.

TIMETABLE.  The authority governing movement of trains subject to the rules.  It contains classified schedules of regular trains and special instructions.

TRAIN.  An engine, or more than one engine, with or without cars, displaying markers.

TRAIN OF SUPERIOR CLASS.  A train given precedence by timetable.

TRAIN OF SUPERIOR DIRECTION.  A regular train given precedence in the direction specified by the timetable as between opposing trains of the same class.

TRAIN OF SUPERIOR RIGHT.  A train given precedence by train order.

TRAIN REGISTER.  A book or form used at designated stations for registering signals displayed, time of arrival and departure of trains and such other information as may be prescribed.

YARD.  A system of tracks within defined limits, other than main track and sidings, provided for the making up of trains, storing cars and other purposes, over which movements may be made without timetable or train order authority, but subject to direction of a yardmaster, if on duty, and such rules, instructions and signals as may be prescribed.

YARD ENGINE.  An engine assigned to yard service.

YARD LIMITS.  The territory between signs placed adjacent to the main tracks to designate the points between which engines may operate on main tracks without train order or timetable authority, and within which engines and certain trains are restricted in their movements on all tracks.

NOTE:  Where, in these rules, special instructions, timetable bulletins, or in train orders, the following terms appear, they will apply as follows:

TRAIN or TRAINS:  In connection with speed restrictions, or the observance of signals (except train-order signals),  also applies to engines.

CONDUCTORS:  To conductor or yard-engine foreman.

TRAINMEN:  To conductors and brakemen, yard engine foremen and switchmen.

ENGINEMAN or ENGINEMEN:  To engineers, firemen, hostlers and hostler helpers.

OUTSIDE OF BLOCK SYSTEM LIMITS:  Also applies to movement on any track which is provided with block signals for movement in one direction only, when movement is being made in the direction for which block signals are not provided.

RULES FOR SINGLE AND DOUBLE TRACK

STANDARD TIME

1.  Standard time is to be obtained from any source authorized in timetable by employee charged with the duty of maintaining standard clocks with the correct time.  Clocks bearing the prescribed sign “standard clock” will be maintained at designated places indicated in timetable, and employee charged with the duty must set clock to correct time at least once each day office is open.

2.  Each of the following employees, and such other employees as may be designated, must carry while on duty a reliable watch:

* Train-order operators                    Trainmen

* Signal operators                            Enginemen

Yardmaster

(*except when assigned in offices where a standard clock is located.)

3.  Employees designated in Rule 2 must, when commencing each day’s work, compare time with a standard clock.  Where standard clock is not available, correct time must be obtained from the train dispatcher, or from an employee designated in Rule 2 who has made such comparison.

Watches must be set to correct time if they reflect a variation of more than 20 seconds from correct time when comparison is made as prescribed by this rule.

TIMETABLES

4.  Each timetable from the moment it takes effect, supersedes the preceding timetable and its schedules take effect on any subdivision at the leaving time at their initial station on such subdivision.  Schedules on each subdivision date from their initial station on such subdivision.  Not more than one schedule of the same number and day may be in effect on any subdivision.

4-A.  Notice of new timetable or supplement must be issued by timetable bulletin.  Notice by train-order, Form “Q”, must be given to conductors and engineers of trains.

4-B.  Special instructions in timetable supersede any rule or regulation of the book of rules with which they conflict.

Special instructions appearing on a schedule page of timetable apply only to the page on which they appear, unless otherwise provided.

Bulletin boards and/or bulletin books will be provided at stations designated by timetable.  Timetable bulletins will contain only information or instructions relating to the rules, or movement of trains; supersede special instructions in the timetable, or any rule or regulation of the book of rules with which they conflict, and expire with the current timetable.  Instructions affecting the superiority of trains must not be issued by timetable bulletin.

Trainmen, enginemen and others concerned in the movement of trains must review timetable bulletins when commencing each day’s work.

Timetable bulletins will be issued by authority and over the name of the Superintendent, and will be numbered consecutively during the effectiveness of each timetable, and bear the name of the division and the number of current timetable.

5.  Not more than two times are given for a train at any station; where one is given, except at terminating stations, it is the leaving time; where two, they are arriving and leaving time.

Unless otherwise provided, time applies at the clearance point of siding where an inferior train must be clear of main track; where there is no siding, it applies at train-order signal; where there is neither siding nor train-order signal, it applies at station sign.

Schedule meeting or passing times are indicated by figures in full-faced type.

Both the arriving and leaving times of a train are in full-faced type when both are meeting or passing times, or when one or more trains are to meet or pass it between those times.

When there are one or more trains to meet or pass a train between the two times, or more than one train to meet or pass a train at any station, it will be indicated by a heavy dash under figures, thus:

5.45   or   7.10  or   7.30

6.  The following symbols when placed before the figures of a schedule indicate:

s – Regular stop,

f – Flag stop to receive or discharge traffic,

c – conditional flag stop as prescribed by special instructions.

6-A.  The following symbols when placed at left of station name indicate:

TO – Train order office.

R – Train register station.

The following symbols when placed at left of page indicate:

B – Bulletin station,

K – Standard clock,

W – Water station,

D – Diesel fuel station,

O – Fuel oil station,

I – Interlocking,

T – Turntable,

Y – Wye or turning track,

P – Telephone.

Double track or CTC limits will be indicated within brackets at the right of station column.

ABS or A-PB limits will be indicated within brackets at the left of station column.

The following abbreviations may be used in conjunction with brackets opposite station column in the timetable:

DT – Double track,

CTC – Centralized traffic control,

ABS – Automatic block signal system,

A-PB – Absolute-permissive block.

SIGNALS AND THEIR USE

7.  Employees whose duties may require them to give signals must provide themselves with the proper appliances, keep them in good working order and ready for immediate use.

7-A.  Yellow flags and unattended red flags and red lights must be placed to the right of the track in the direction of approach.  When practicable, all signals by hand must be given on engineer’s side.

NOTE:  Flags may be of cloth, metal or other suitable materials.

10. COLOR SIGNALS

COLOR 		INDICATION

a. Red			Stop.

b. Yellow		Proceed at reduced speed, and for other uses prescribed by the rules.

c. Green		Proceed, and for other uses prescribed by the rules.

d. Green and White	Flag stop.  See Rule 28.

e. Blue			See Rules 26 and 297.

10-G.  When an unattended red flag or red light is displayed on or near the track, train, after stopping, must be preceded for a distance of three-fourths (3/4) mile from point where signal is displayed, by a flagman who must carefully examine track and structures.

10-J.  Speed signs will be located to the right of track in direction of approach where practicable.  On double track where trains keep to left, speed signs will be located to left if proximity of adjacent main track prevents locating to right.

Speed signs that prescribe reduction in speed will be located one-half (1/2) mile from the initial point of restriction, and where used to authorize increase in speed will be located at point where higher speed commences.  Speed may be increased as soon as rear of train has passed the speed sign.  Where such signs are not used to authorize an increase in speed, limit of restriction will be shown in timetable.

The higher number on speed sign indicates the maximum speed for trains consisting entirely of passenger equipment, and the lower number indicates the maximum speed for all other trains.  Where but one number is shown, it indicates the maximum speed for all trains.

Round yellow speed signs indicate by black figures the maximum speed for certain passenger trains designated by special instructions in the timetable.  Speed indicated by oval white speed signs applies to those trains unless a round yellow speed sign is displayed on same post below the oval speed sign.

12.  HAND SIGNALS FOR TRAIN AND ENGINE MOVEMENTS (with hand, flag or lantern.)

MANNER OF USING								INDICATION

A. Swung at right angle to track.							Stop

B. Slight horizontal movement at arms length						Reduced Speed

at right angle to track.

C. Raised and lowered vertically.							Proceed, also train-

men’s answer to

signal 14(k).

D. Swung vertically in a circle at right angle to track.					Back

E. Swung horizontally above the head at right angle to track when standing			Apply air brakes

F. Held at arm’s length above the head when standing					Release air brakes

12-A.  Any object waved violently by anyone on or near track is a signal to stop.

12-B.  Radio, telephone or other means of voice communication may be used in place of hand signals to convey information.

14.  ENGINE WHISTLE SIGNALS

SOUND		INDICATION

(a)   o			Apply brakes. Stop

(b)   -- ooo		Flagman protect rear of train.

(c)   ooo --		Flagman protect front of train.

(d)   -- -- -- --		Flagman may return from west.

(e)   -- -- -- -- --		Flagman may return from east.

(f)    -- -- --		Train parted.

(g)   oo			Answer to 14(k) or any signal not otherwise provided for.

(h)   ooo			When standing, back.  Answer to 12(d)

(j)   oooo		Call for signals.

(k)   -- oo                            (Single track) To call attention of engine crews and train crews of trains of the same class, inferior trains and yard engines, and of trains at train-order meeting or waiting points, to signals displayed for a following section, unless otherwise provided by special instructions.  Failure to receive acknowledgment by signal 14(g) and 12(c) must be reported.  Signal 14(k) also to be sounded when passing rear of freight train.

(Double track)  To call attention of engine crews and train crews of trains of same class and of inferior trains and yard engines, to signals displayed for a following section.

(l)   -- -- o --	                     Approaching public crossing at grade, tunnels and obscure curves; to be commenced sufficiently in advance to afford ample warning, but not less than one-fourth (1/4) mile before reaching a crossing, and prolonged or repeated until engine has passed over the crossing.

17.  When engine is so equipped, the headlight must be displayed to the front of every train by day and night, but will be extinguished when meeting trains under the following conditions, where practicable:

If train is clear of main track and has stopped.

When standing at end of double track or at a junction.

When standing on main track if switch has been lined for the opposing train.

Oscillating white light on engines so equipped must be operated at the same time headlight is required to be displayed.

19.  A marker must be continuously displayed while a train is authorized.

20.  All sections, except the last, will display green signals on the lead engine.

20-A.  Extra trains will display white signals on the lead engine.

NOTE:  Trains authorized to display signals must notify inferior trains that are met or passed, including trains at train-order meeting points.  Such notification will be accepted as evidence of green signals displayed.  Identification of engine authorized to run extra will be accepted as evidence of white signal displayed.

21.  Trains must be identified by number on lead engine when practicable.  When not practicable, trains will be identified by number of another engine on the head end of the train.

Engine number of regular trains and sections thereof, when necessary, must be furnished by train order, message or verbal instructions to all trains that are to be met or passed.

22.  When two or more engines are coupled at the head of a train, the leading engine only will display signal; except when road engine is coupled behind a helper over part of a subdivision, the road engine will display signals the same as displayed on helper engine.  The lead engine only will give or answer signals.

26.  A blue signal displayed at one or both ends of an engine, car or train, or suspended from a mast placed in center of track, indicates that workmen are under the engine, car or train.  When so displayed the engine, car or train must not be coupled to or moved.

27.  A signal improperly displayed, the absence of a fixed signal at a place where a signal is usually shown, or absence of a flag or sign where required, must be regarded as the most restrictive indication that can be given by that signal, except that when day indication, if any, is plainly seen, it will govern.  Imperfectly displayed or absence of fixed signal must be promptly reported to train dispatcher.

28.  A green and white signal must be used to stop a train at authorized flag stops.

29.  When a signal is given to stop a train, it must be acknowledged and unless proceed signal is received, reason for stopping train must be made known to engineer before train proceeds.

29-A.  When cause for a flagman’s signal is fully explained to engineer, and circumstances do not require train to stop, it may proceed, being governed by instructions of flagman.

30.  Engine bell must be rung when engine is about to be moved; while passing through tunnels; while approaching public crossings at grade, beginning sufficiently in advance to afford ample warning, and continuing until engine has passed over crossing.

32. Unnecessary use of either whistle or bell is forbidden.

SUPERIORITY OF TRAINS

70.  Within limits in which Rule D-251 is applicable, and within interlocking, A-PB and CTC, signal indications supersede the superiority of trains.

S-71.   A train is superior to another train by right, class or direction.  Right is conferred by train order; class and direction by timetable.  Right is superior to class or direction.  Direction is superior between trains of the same  class.

72.  First-class trains are superior to second-class, second-class are superior to third-class, and so on.

S-72.   Regular trains in the direction specified by timetable are superior to trains of the same class in the opposite direction.

73.  Extra trains are inferior to regular trains.

Second and following sections of a schedule are inferior to preceding sections of the same schedule.

MOVEMENT OF TRAINS

81.  Main track must not be occupied without authority.  Outside of block system limits main track must not be fouled until by observation or flag protection the engineer or conductor, as the case may be, is assured it is safe to do so.  In block system limits Rule 513 must be complied with before fouling main track.

82.  Timetable schedules, unless fulfilled, are in effect for three (3) hours after their time at a station.

Regular trains more than three (3) hours late on either their schedule arriving or leaving time at any station lose both schedule and train-order authority, and can thereafter proceed only as authorized by train order.

82-A.  Unless otherwise provided, regular trains will be authorized at their initial station by clearance, which must bear the OK, time and initials of the chief train dispatcher.  At an intermediate station, following form of train order must be used:

“ENGINE _____ RUN AS NUMBER _____ FROM _________”

When its initial station is an open train-order office, unless otherwise provided, an extra train must not leave without clearance, which must bear the OK, time and initials of the chief dispatcher.

83.  A train must not leave its initial station, or a junction, or an intermediate station where schedules originate or terminate, or pass from double track to single track, or from CTC or interlocking to other track, until it is known that all superior trains due have arrived or left, or that it has authority to proceed.

Visual or oral identification, register check or train-order check of a section, will be evidence that all preceding sections of the same schedule have also arrived.

Stations at which train registers are located will be designated in timetable.

83-A.  Information called for by train register must be inscribed therein and the register checked by the conductor, or by engineer if there is no conductor, except as herein provided.

Regular trains will register their arrival on pages of the register for the day on which such regular trains are due to arrive; and register their departure on pages for the day on which such trains are due to depart.

An extra train will register only at a register station where it originates or terminates, unless otherwise directed.

All lines on page of register must be filled in before starting a new page for the same day.

83-B.  A train may leave a register ticket on prescribed form with the train-order operator at train-register station when authorized by special instructions or by train order, or when train-order check of trains is received.  When so authorized, the operator must enter on the register information contained on the ticket, then report train from register.

83-D.  When a train is restricted for another train, the restricted train must not leave until train has been identified, or train order is received superseding or annulling the restriction, or train order Form V is received.

A train may check register against an extra.

84.  A train must not start until the proper signal is given.

85. When a train of one schedule is on the time of another schedule of the same class in the same direction, it will proceed on its own schedule.

Trains of one schedule may pass trains of another schedule of the same class.  Second and inferior class and extra trains may pass and run ahead of second and inferior class and extra trains.

Unless otherwise provided a section must not pass and run ahead of another section of the same schedule without first exchanging train orders, and each assuming the other section’s number displaying signals if necessary.  The change in sections must be reported from the first available train-order office.  If trains are met or passed after change is made, sections involved must stop and identify themselves to such trains, unless train order Form F, example 5, is issued to trains involved.

86.  Except within interlocking, A-PB, CTC and territory where Rule D-251 applies, a train must be clear of the main track:

(a) Not less than two (2) minutes before the leaving time of an opposing superior train;

(b) Before a superior first-class train or a train of superior right in the same direction is due to leave the next station to the rear where time is 	      shown.

S-88.  Extra trains will be governed by train orders with respect to opposing extra trains.

At a train-order meeting point between extras, train in inferior timetable direction must take siding, unless train order provides otherwise.

89.  When an inferior train fails to clear a superior train by the time required by rule, it must be protected at that time as prescribed by Rule 99.

Trains required to take siding must head in, when practicable.  If necessary to pull by and back in, or enter siding at other than the initial switch, train must be protected by flagman before movement is made beyond the initial switch, unless authority for occupancy of main track is otherwise provided.

S-90.  At meeting points trains must stop short of fouling point, and when practicable, must stop back not less than 400 feet from fouling point.

91.  Outside of block system limits, trains in the same direction must keep not less than two (2) minutes apart.

92.  A train must not leave a station in advance of its schedule leaving time.

93.  Within yard limits, engines, after complying with provisions of Rule 81 or Rule 513, may use main track without train-order authority, clearing against first-class trains, and without flag protection against second and inferior-class trains, extra trains and engines.

Second and inferior-class trains, extra trains and engines must move with caution on main track within yard limits, except where movements are governed by block signal indication.

Trains and engines must not move against the current of traffic within yard limits until provision has been made for protection of movement, except where movement is within interlocking limits and protection is afforded by interlocking signals.

95.  Two or more sections may be run on the same schedule.  Each section has equal timetable authority.

A train must not display signals for a following section without train-order authority, except as prescribed by Rule 85, or upon receipt of clearance of initial station with the words “green signals” following the section number.

96.  Clearance or train-order requiring display of signals authorizes such signals to be displayed to the terminal on the subdivision.  A train order must not be issued creating a section at an intermediate point or to take down signals at an intermediate point of schedule.  When it is desired to discontinue the last section from initial station or any intermediate point, train-order form K must be used.

97.  Unless otherwise provided, extra trains must not be run without train-order authority.

D-97.  When authorized by special instructions in timetable, trains moving with the current of traffic may run extra or work extra without train-order authority, but must obtain a clearance before commencement of trip if at an open train-order office.

98.  Trains and engines must approach end of double track, junctions, railroad crossings at grade and drawbridges, prepared to stop, unless switches are properly lined, track is clear and signals, if any, display proceed indication.

99.  When a train is moving under circumstances in which it may be overtaken by another train, the flagman must take action as may be necessary to insure full protection.

Except as prescribed in Rule 99-A, when a train stops under circumstances in which it may be overtaken by another train, the flagman must go back immediately with flagman’s signals a sufficient distance to insure full protection.

The front of the train must be protected in the same way when necessary.

Conductors and engineers are responsible for the protection of their trains or engines.

99-A.  When rear of train is standing within yard limits, flag protection to the rear is not required against second and inferior-class trains, extra trains and engines.  When a train stops within block system limits, with protection of at least two block signals to the rear, flagman must go back immediately with flagman’s signals a sufficient distance to insure full protection against following trains moving at restricted speeds, except when rear of train is standing between opposing absolute signals at a station, or is within interlocking limits, flag protection to the rear is not required.

S-99-B.  When a flagman is sent with specific instructions restricting the movement of a train or engine, such instructions can be oral or written.  When such flagging is to permit movement against an opposing train, the flagman must stop all opposing trains.  He must show flagging instructions to the engineer of train or trains so restricted.

99-C.  When a regular train, in territory designated by Superintendent, receives train order, Form I, Example (1), protection against extra trains in direction specified is not required until the time named.  Extra trains in direction specified must not follow the regular train until time named.

When an extra, in territory designated by Superintendent, receives train order, Form I, Example (2), protection against extra trains in direction specified is not required until time named.  Other extra trains in direction specified must not enter territory specified until the time named.

101-A.  Instructions respecting movement of trains or condition of track or structures must be in writing, except within yard limits where movements are subject to yardmaster’s instructions, within CTC where movements are subject to train dispatcher’s instructions and within interlocking where movements are subject to operator’s instructions.

102.  If a train should part while in motion, trainmen must take every precaution to prevent damage to the detached portions.

102-A.  When portion of train is left on main track where view is obscured, it must be protected.

104.  Except where switch tenders are in charge of switches, trainmen and enginemen are responsible for proper setting of hand-operated switches and derails to be used by their train or engine, and for their return to proper position after use.

A train or engine must not foul a main track until switches connected with the movement are properly lined.

Where trains or engines are required to be reported clear of main track, such report must not be made until switch has been returned to normal position.

A switch must not be left open for another train or engine unless there is a thorough understanding with the other train or engine of the movement being made.

When spring switches or dual control switches are operated by hand, they are considered hand-operated switches within the meaning of this rule.

104-B.  Rigid switches must not be run through.  A rigid switch that has been run through must be inspected, and if found unsafe must be spiked and the matter reported to the proper authority.

104-C.  When switch is lined, employee setting it must see that both points have moved to proper position.

Crossover switches must not be opened when there is a train, engine or car closely approaching either switch.  When crossover movement is to be made, both switches must first be opened before movement is commenced, and within block system limits movement must be completed before either switch is restored to normal position.

104-F.  Within interlocking, CTC or A-PB limits where main track hand-operated switches are not equipped with electric or mechanical switch locks and maximum speed is in excess of 20 MPH, trains and engines using such switches must occupy main track continuously or leave main track switch open while work is being performed. Such tracks must not be used by trains or engines for meeting or passing of trains.

105.  Trains or engines using other than main track must proceed with caution.

Sidings must not be blocked unless authorized by train dispatcher, except in emergency.  When so blocked, report must be made to the train dispatcher from the first open train order office.

Sidings of an assigned direction must not be used in a reverse direction unless authorized by the train dispatcher, or in an emergency under full protection.

When practicable, a train entering a siding will not stop until train is clear of the main track.

106.  The conductor and the engineer, and pilot if any, are responsible for the safety of the train and observance of the rules, and, under conditions not provided for by the rules, must take every precaution for protection.

107.  When a passenger train is receiving or discharging traffic at a station, a train or engine must not pass between it and the station platform unless proper safeguards are provided, and movement must be made with caution.

108.  In case of doubt or uncertainty, the safe course must be taken.

D-151.  Trains and engines must keep to the right, unless otherwise provided.

D-152.  Before a train or engine crosses over to or obstructs another main track, it must first be protected on that track

RULES FOR MOVEMENT BY TRAIN ORDERS

201.  For movements not provided for by timetable, unless otherwise provided, train orders will be issued by authority and over the initials of the chief train dispatcher and only contain information or instructions essential to such movements.

They must be brief and clear; in the prescribed forms when applicable; and without erasure, alteration or interlineations.

Figures in train orders must not be surrounded by brackets, circles or other characters.

202.  Each train order must be given in the same words to all employees or trains addressed.

203.  Train orders must be numbered consecutively each day, beginning at midnight.  Duplicate numbers of the same date must not be used over the initials of the same chief dispatcher.

204.  Train orders must be addressed to those who are to execute them, naming the place at which each is to receive his copy.  Those for a train must be addressed to the conductor and engineer, and also to the pilot, if any.

Orders addressed to operators restricting the movement of trains must be respected by conductors and engineers the same as if addressed to them.

Conductors and engineers must read train orders, clearances, see that the order numbers shown on clearance correspond with the number of orders received, and call attention to errors or omissions.  They must compare their understanding of the orders and remind each other of their contents.

205.  Each train order must be written in full in a book provided for the purpose in the office of train dispatcher; and with it recorded time and the signals which show when, and from what office order was repeated and the responses transmitted.  The date and train dispatcher’s initials must be placed at the top of each page.  These records must be made at once and never from memory or memoranda.

Additions to train orders must not be made after they have been repeated.

If an error is made in transmitting a train order it must be immediately destroyed, and the same order number must not be again used on the same day.

206.  In train orders and clearances regular trains will be designated by number, thus:  NO 58, and sections, thus:  SECOND 55.  Extras will be designated by engine number and the direction, thus:  EXTRA 6236 EAST.  Work extras will be designated by engine number, thus:  WORK EXTRA 2718.

When an engine of another railroad is used, the initials of that road must precede the engine number, thus:

EXTRA ATSF 269 EAST.

Even hours must not be used in stating time of day in train orders.

206-A.  In transmitting and repeating train orders and clearances, names of stations, sections and directions of extras must be plainly pronounced, and then spelled, letter by letter, thus:  AURORA, A-U-R-O-R-A; SECOND, S-E-C-O-N-D; EAST, E-A-S-T.  Order numbers, train, engine, and other numbers, and time, must first be pronounced and followed by pronouncing each figure, thus:  One hundred five, 1-0-5; Twenty seven fifty-six, 2-7-5-6; Nine fifty, 9-5-0.  Where the number is but one figure it must first be pronounced, thus:  One, figure 1, then spelled, thus: O-N-E.

The names of stations, sections, direction of extras, order numbers, train, engine, and other numbers, and time, must be written in train orders and in train order book, thus:  AURORA; SECOND; EAST; 105; 2756; 950; 1.

206-B.  When transmitting a train order, the dispatcher must write it in the train-order book as he transmits the order and underscore each word and number as repeated by each office.

206-C.  To relay a train order, it must be transmitted in the usual manner to the relaying office.  Operator at relaying office must transmit the order to destination.  Employee receiving the order at destination must repeat the order to operator at relaying office, who must underscore on his copy each work and number as repeated.  He must then repeat the order to train dispatcher, by whom “complete” will be given to relaying operator, who will transmit it to destination.

If order is also addressed to a superior train at relaying office, the “X” response must be given before it is transmitted to the inferior train.

207.  To transmit a train order, the direction must be given to each office addressed, and the number of copies stated.

208.  A train order to be sent to two or more offices must be transmitted simultaneously to as many of them as practicable.  When not sent simultaneously to all, the order must be sent first to the superior train.  The several addresses must be in the order of superiority of trains, each office taking its proper address.

A train order restricting movement of a train must not be issued for it at the station where such movement is restricted if it can be avoided.  When this cannot be avoided, and the order is sent to a point other than its initial station or a station within interlocking limits, the following must be added to the order:

ORDER TO ______ AT ______.

Opposing trains that have received this order must take siding if the restricted train is met at the station named, unless order otherwise provides.

Train dispatcher must not give OK to the clearance until assured by the operator that the train has stopped.

208-A.  When a train has received a clearance, or the engine of which has passed train-order signal displaying proceed indication, a train order restricting the superiority of that train must not be repeated, nor “X” response sent until conductor or engineer is in the train-order office and has assured operator that he understands that train is to be restricted.

If clearance has been received at that station, all copies of such clearance must be returned to the operator and destroyed before new clearance and the restricting order are delivered.

208-B.  When a train order restricting the superiority of a train is to be issued direct to a train, train dispatcher must have a personal understanding with conductor or engineer that train is to be restricted before transmitting order.

Before transmitting a train order direct to a train, the person who is to receive the order must give his name, train identification and location.

The “X” response must not be used when handling train orders direct to a train.

209.  Operators receiving train orders must write or typewrite them in manifold during transmission.  When necessary to make additional copies of a train order, operator, after repeating new copy to train dispatcher, must sign his own name on new copies.  Train dispatchers must record time and date, and name of office repeating each recopied train order.

210.  When a train order has been transmitted, unless otherwise directed, operators must repeat it in the succession in which the several offices have been addressed.  Each operator receiving the train order must observe whether others repeat correctly, unless relieved of this duty by train dispatcher.  After each order has been repeated correctly, the response “complete”, time, and initials of chief train dispatcher will be given by the train dispatcher.  Operator will then write or type, in space provided, the time and his last name.

212.  When directed by train dispatcher, a train order, having been transmitted, may be acknowledged by operator as a holding order until repeated, by responding:  “______ (order number) ______ to ______ (train number) ______X”, with the operator’s initials and office name.  Operator must then write or type on the order his initials and time.

213.  “Complete” must not be given to a train order for delivery to an inferior train until order has been repeated or “X” response sent by operator who receives order for superior train.

214.  When a train order has been repeated, it must be completed at once, but when “X” response is sent, and until it has been completed, order must be treated as a holding order for train or trains addressed, and must not be otherwise acted on until “complete” has been given.

If communication fails before an office has repeated an order or has sent the “X” response, the order is of no effect and must be there treated as if it had not been sent.

216.  When an order is to be issued and delivered to a train by the train dispatcher, a carbon impression must be made in the train-order book at the time the order is written.  If later the order is to be sent to another office, the requirements for delivery are the same as at other offices.

217.  A train order to be delivered to a train at a point not a train-order office, or at which the office is closed, must be addressed to “C&E ______ AT ______ CARE OF ______”, and forwarded and delivered by the employee in whose care it is addressed.  When sent in care of conductor or engineer of a train, the number of the order must be shown in the usual manner on clearance for train making delivery the same as if addressed to it.  Copies of order must be provided for conductor and engineer of train making delivery and copies for conductor and engineer of train being addressed.

Orders must not be sent, in manner herein prescribed, to a train, the superiority of which is thereby restricted.

Train orders for use by the same crew on another trip are not to be addressed to the train by the identification it will have on a later trip, but by the identification it is authorized to display when the train orders are received.

Train orders must not be addressed to a crew in care of themselves.

218.  When a schedule is designated in a train order by its number alone, all trains operating as sections of that schedule are included, and each must have copies delivered to it.

This does not apply to addresses on clearance.

220.  Train orders once in effect continue so until fulfilled, superseded or annulled.  Any part of an order specifying a particular movement may be either superseded or annulled.

Orders held by or issued for, or any part of an order relating to, a regular train, become void when such train loses both right and schedule as prescribed by Rule 4 and Rule 82, or its schedule is annulled.

When the authority for an extra or work extra is fulfilled or annulled, other orders held by, or any part of an order related to such extra or work extra becomes void unless otherwise provided.

When a crew is relieved before the completion of a trip, all train orders, clearances and instructions held must be delivered to the relieving crew.

220-A.  Train orders related to track conditions, unless annulled, must be respected by conductor and engineer on all trips made during the tour of duty on which such orders are received.

A train order received by a train may be made applicable to an additional trip by issuance of an order reading “RESPECT ORDER NO. ______”, or adding these words to the order creating the train for the additional trip.

221.  A train-order signal will be provided at each train-order office, except at those where all trains are required to obtain clearance.  Light-type train-order signal will be designated by letters “TO” on the mast.

The arm of a train order signal to the right, as seen from an approaching train, is the one which governs.  When the arm is extended horizontally, or in addition a red light is displayed, it indicates “STOP”, and when slanted downward at an angle of 60 degrees, or in addition a green light is displayed it indicates “PROCEED”.  Arms will be positioned to indicate “PROCEED” when a train order office is closed.

When red light is displayed in light-type train-order signal it indicates “STOP”.   When green light is displayed it indicates “PROCEED”.  Green light will be displayed when train order office is closed.

Train-order signal must be kept in stop position for both directions while operator is on duty, except signal will be cleared for approaching train when no orders are held for any train in the same direction other than for trains originating.

Signal must be restored to stop position as soon as practicable after rear of train has passed, and operator must not OS the train, nor “X” or repeat an order for a following train until signal has been restored.  If signal is restored to stop position before rear of train has passed the signal, train must stop and obtain clearance.

When signal remains in stop position for an approaching train, a clearance must be obtained, and engine must not pass the fouling point of the switch at which an opposing train may enter the siding, until engineer has ascertained that he is not to receive train orders which restrict his train at that point.

Delivery of orders while train is moving will indicate that orders do not restrict the train at that station.

Outside of block system limits and for movements into territory outside of block system limits, operators must space trains two minutes apart.  When clearance is delivered before the expiration of two minutes, operator must show on clearance the time following train may leave.

221-A.  Before clearing a train for which there are orders, operator must carefully read the address of each order held, fill out the clearance, showing thereon without erasure or alteration, number of each train order addressed to train or in care of an employee on the train, then transmit address and order numbers from clearance to train dispatcher, who must check correctness thereof against his record in train-order book, and if correct give the OK, time and initials of chief dispatcher, and make proper record thereof.  Operator, after entering this information on clearance, may make delivery.

When necessary to issue clearance to a train for which there are no orders, the word “No” must be written in space provided for number of orders and time issued will be inserted following operator’s name.  Such clearance will not bear dispatcher’s OK, except when used to authorize a train at its initial station as prescribed by Rule 82-A.

Clearances may be handwritten or typewritten, and must bear date of issuance, with time in the proper space.

If, after a clearance is delivered to conductor or engineer, or both, orders are issued for a train which do not restrict its superiority, train dispatcher, without requiring operator to destroy the clearance first issued, may authorize issuance of a second clearance which must be endorsed “second” preceding the word “clearance” on top margin.  All orders for delivery to train at that station must be shown on second clearance.

222.  Operators must promptly record, and report to train dispatcher, the arrival and departure of all trains, reporting “no signals” or “green signals”, as the case may be.  If report of signals displayed is incorrect, train dispatcher must immediately notify all trains affected until he has ascertained that signals are properly displayed, and should “no signals” be registered when “green signals” should have been registered, he must require immediate correction in train register.

222-A. Operators in relieving each other must make a transfer of undelivered train orders and messages addressed to, or in care of trains; also show on the transfer numbers of all overdue trains and whether trains then in the yard or at stations have or have not been cleared.

A train-order office must not be closed unless permission is obtained from the train dispatcher.

222-B.  Train dispatchers in relieving each other must make a written transfer in the train order book of all train orders and clearances in effect.

The transfer must include special mention of any unusual conditions.

The relieving dispatcher must read and check each train order and clearance in effect.

Transfer must then be signed by both train dispatchers and the time transfer is made must be shown.

223.  Following signals and abbreviations may be used:

Initials for name of chief train dispatcher, such office and other signals as are arranged for by superintendent.

C&E	 Conductor and engineer,

X	 Train will be held until train order is made complete,

COM	 complete,

OK	 correct,

OS	 train report,

NO	 number,

ENG	 engine,

PSGR	 passenger,

FRT	 freight,

MINS	 minutes,

JCT	 junction,

DIV	 division,

SUBDIV subdivision,

MP	 mile post,

MPH	 miles per hour,

DISPR	 dispatcher,

OPR	 operator,

YM	 yardmaster,

ENGR	 engineer,

CONDR conductor

The following for names of the month:  Jan, Feb, Mar, Apr, May, June, July, Aug, Sept, Oct, Nov, Dec.

FORMS OF TRAIN ORDERS

Note:  Forms with prefix “S” are for single track; those with prefix “D” are for double track; those without prefix are for single or double track.

S-A

Fixing Meeting Points for Opposing Trains

(1) NO 51 ENG 4443 MEET NO 4 ENG ATSF 17 AT ILMON

(2) NO 55 ENG 4217 MEET SECOND 4 ENG ATSF 3751 AT CALIENTE

(3) NO 57 ENG 6392 MEET EXTRA 6236 EAST AT ALLARD

(4) EXTRA 352 EAST MEET EXTRA ATSF 269 WEST AT CALIENTE

(5) NO 52 ENG 4456 AND SECOND 4 ENG ATSF 329 MEET NO 55 ENG 4171 AND NO 23 ENG ATSF 51 AT ALLARD AND EXTRA 4287 WEST AT BEALVILLE

(6) NO 52 ENG 4352 MEET NO 55 ENG 4284 NO 23 ENG ATSF 40C AND NO 7 ENG ATSF 3756 AT CLIFF

(7) NO 51 ENG 4431 MEET FIRST 58 ENG 6390 AND SECOND 58 ENG 4356 AT ROWEN THIRD 4 ENG ATSF 3456 AND FOURTH 4 ENG ATSF 3768 AT CLIFF AND EXTRA 4294 EAST AT BEALVILLE NO 51 TAKE SIDING AT ROWEN AND BEALVILLE

(8) FIRST 55 ENG 4264 SECOND 55 ENG 5333 AND THIRD 55 ENG 4368 MEET NO 58 ENG 4294 AT WOODFORD

(9) SECOND 447 ENG 6234 HOLD MAIN TRACK MEET NO 52 ENG 4347 AT ILMON

(10) SECOND 57 ENG 4350 MEET NO 806 ENG 5353 AT ALLARD AND NO 448 ENG 6448 AT CALIENTE NO 806 HOLD MAIN TRACK AT ALLARD

When there is more than one meeting point made in a train order, and it is desired that superior train take siding; or that inferior train hold main track at either, or all, of any meeting points, such instructions must be in a separate paragraph in the order.

Trains receiving form S-A train orders will run with respect to each other to designated points and there meet in the manner prescribed by the rules, and unless designated train is met, will not proceed until further authority to move is given by train order.

Work extras must not be given meeting points with each other, nor with other trains.

B

Directing a Train to Pass or Run Ahead of Another Train

(1) NO 447 ENG 5490 PASS NO 55 ENG 4242 AT BEALVILLE NO 55 TAKE SIDING

Both trains will run according to rule to the designated point and there arrange for the following train to pass promptly.  Order must state which train is to take siding.  When an inferior train receives an order to pass a superior train, authority is conferred to run ahead of train passed, from designated point.  If after leaving the station named, first-named train is delayed, it may allow second-named train to pass and authority to run ahead is void.

(2) EXTRA 6290 EAST RUN AHEAD OF NO 58 ENG 6054 BAKERSFIELD TO ALLARD

First-named train will run ahead of second-named train between points designated.

If a train is delayed after receiving an order to run ahead of a superior train, first-named train may allow second-named to pass, and authority to run ahead will then become void.

S-C

Giving Right Over an Opposing Train

(1) NO 448 ENG 5624 HAS RIGHT OVER NO 57 ENG 6441 BENA TO TEHACHAPI

If second-named train reaches point last named before the other arrives, it may proceed, clearing time of the opposing train as required by rules.

(2) EXTRA ATSF 122 EAST HAS RIGHT OVER NO 55 ENG 5320 BENA TO CALIENTE

Regular train must not go beyond point last named until extra train has arrived, unless authorized by train order.

Examples (1) and (2) give right to train first named over regular train between points named.  If trains meet at either of designated points, first-named train must take siding, unless order otherwise prescribes.

- - - - - - - - - - - - - - -

(3) EXTRA 4287 WEST HAS RIGHT OVER EXTRA 6290 EAST TEHACHAPI TO ALLARD AND WAIT AT MARCEL UNTIL 330 PM

WOODFORD 350 PM

CLIFF 420 PM

FOR EXTRA 6290 EAST

This order gives right to first-named extra over other extra named between points designated, but not at either of those points, and does not restrict first-named train beyond initial switch of siding at last-named station.

If second-named extra exists beyond second-named point at time the order is issued, the following must be added to the order:  AND TAKE SIDING NOT LEAVING ALLARD UNLESS EXTRA 6290 EAST HAS ARRIVED

D

Giving Right Over Another Train in the Same Direction

(1) NO 23 ENG ATSF 32 HAS RIGHT OVER NO 55 ENG 4264 MOJAVE TO BAKERSFIELD

Second-named train must clear time of first-named train as prescribed by Rule 86.

- - - - - - - - - - - - - - -

(2) EXTRA 6345 WEST HAS RIGHT OVER NO 59 ENG 4347 MOJAVE TO CALIENTE AND WAIT AT

MOJAVE UNTIL 250 PM

CLIFF	 	255 PM

ALLARD 	301 PM

(3) EXTRA ATSF 200 WEST HAS RIGHT OVER EXTRA 4137 WEST MOJAVE TO BAKERSFIELD

Examples (2) and (3) give right to first-named train over other train between points named.  If train-order time is given, first-named train must not pass designated stations before time shown, and second-named train must clear such time as prescribed by Rule 86.  If no time is given, second-named train must not leave first-named point ahead of first-named train.

E

Time Orders

(1) NO 51 ENG 6459 RUN 50 MINS LATE MOJAVE TO BAKERSFIELD

(2) NO 51 ENG 4457 RUN 50 MINS LATE MOJAVE TO CALIENTE AND 20 MINS LATE CALIENTE TO BAKERSFIELD

This form of order makes the schedule time of a train named, between stations named, as much later as stated in order.  Any other train receiving this order is required to run with respect to this later time, as before required to run with respect to schedule time.  Time in this order should be such as can easily be added to schedule time, as 10, 20, etc.

- - - - - - - - - - - - - - -

(3) NO 24 ENG ATSF 168 WAIT

BENA UNTIL	120 PM

CALIENTE	130 PM

BEALVILLE	140 PM

Train, or trains, named must not pass the designated points before the time given.  Other trains receiving the order are required to run with respect to the time specified at designated points or any intermediate station where schedule time is earlier than time specified in the order, as before required to run with respect to schedule time of train, or trains, named.

Station names must be written in column formation.

- - - - - - - - - - - - - - -

(4) NO 52 ENG 4411 RUN 20 MINUTES LATE ON ORDER NO 17

This order makes the time mentioned at each station in an order issued under example (3) as much later as specified and trains receiving this order are required to run with respect to this later time, as before required to run with respect to time specified in order under example (3).

(5) NO 52 ENG 4356 RUN 20 MINS LATE ON ORDER NO 17 FROM CALIENTE

If it is desired that an order to run late on a wait order shall not apply to time given at all of the stations, example (5) may be used, and will have the same meaning except that it applies only to station named in example (5) and succeeding stations.

Examples (4) and (5) may be used in connection with a wait order of an extra when issued in form of example (3).

Examples (1), (2), (3), (4) and (5) may be used in connection with an extra created by example (3) form G, and times at each point stated in that example have the same meaning as schedule times in foregoing examples.

S-E

(1) NO 57 ENG 4193 WAIT AT CLIFF UNTIL  430 PM FOR NO 448 ENG 5353

Train first-named must not pass the designated point before the time given, unless the other train has arrived.  Train last-named is required to run with respect to time specified, at designated point or any other intermediate station where schedule time is earlier than time specified in the order, as before required to run with respect to schedule time of train first-named.

F

For Sections

Sections will be created at their initial stations by clearance bearing the words “green signal”, or “no signals”.  Sections may be created, withdrawn or reversed at an intermediate station by using one of the following examples:

(1) NO 23 ENG ATSF 3751 DISPLAY SIGNALS TEHACHAPI TO BAKERSFIELD FOR ENG 6053

(2) SECOND 58 ENG 2486 DISPLAY SIGNALS CALIENTE TO MOJAVE FOR ENG ATSF 40

Engine named in example (1) will become Second 23 and engine named in example (2) will become Third 58 from station named.

- - - - - - - - - - - - - - -

(3) ENG 5623 DISPLAY SIGNALS AND RUN AS SECOND 4 CALIENTE TO MOJAVE FOLLOWING SECTIONS CHANGE NUMBERS ACCORDINGLY

This example may be used to add any section other than the last section at an intermediate station.

Engine named will display signals and run as directed, and following sections will take next higher number.

- - - - - - - - - - - - - - -

To drop any section other than the last, when there are more than two sections, example (4) will be used:

(4) ENG 6448 IS WITHDRAWN AS SECOND 56 AT WOODFORD FOLLOWING SECTIONS CHANGE NUMBERS ACCORDINGLY

Engine named will drop out at station named, and following sections will take next lower number, last section authorized will not display signals.

Under examples (3) and (4) it is the duty of train dispatcher to know that each conductor and engineer has in his possession all train orders affecting his train in its new position.

To pass one section by another, example (5)  will be used:

(5) ENGS 6390 AND 4368 REVERSE POSITIONS AS SECOND AND THIRD 58 WOODFORD TO MOJAVE

Conductors and engineers of the trains addressed must personally exchange orders, and arrange for display of signals accordingly.  Following sections, if any, need not be addressed.  Engine 6390 will then become Second 58 displaying signals, and engine 4368 will become Third 58 displaying signals if Third 58 displayed signals into first named station.

- - - - - - - - - - - - - - -

Each section affected by these orders must have copies and must arrange signals accordingly.

G

Extras

(1) ENG 4137 RUN EXTRA MOJAVE TO BAKERSFIELD

(2) ENG 2718 RUN EXTRA MOJAVE TO ILMON AND RETURN TO CALIENTE

When example (2) is used extra must go to station second named before returning to station last named, unless order is annulled.  If  second named station is an open train-order office, a clearance ok’d by train dispatcher must be obtained.  This form of order does not relieve train of providing protection by flagmen to the rear in either direction, when required by the rules.

- - - - - - - - - - - - - - -

(3) ENG 4415 RUN EXTRA LEAVING MOJAVE SATURDAY FEB 21 AS FOLLOWS WITH RIGHT OVER ALL TRAINS

LEAVE	  MOJAVE	   1120 AM

CLIFF		   1125 AM

CALIENTE	   1135 AM

ARRIVE  BAKERSFIELD  1145 AM

This order may be varied by specifying kind of extra and particular trains over which the extra shall, or shall not, have right.  Trains over which the extra is given right must clear time of extra as prescribed in Rule 86.  Work extras, whether required to protect or not protect against extras, must clear time of this extra in the same manner.

This order does not relieve extra from compliance with Rule 99.  It must not be issued while an order, example (8)

Form H is in effect, and must not be combined with any other form of order.

(4) AFTER 210 PM ENG 3765 RUN EXTRA CALIENTE TO MOJAVE

Extra must not leave the station first named before the time stated.  This example must not be used in lieu of other forms of orders which provide protection against opposing extra work trains.

When an order, Form G, is fulfilled or annulled, other forms of orders held by extra so created, except Forms K, V, and Y and train orders pertaining to track conditions, become void.

(5) AFTER EXTRA 6236 EAST ARRIVES CALIENTE ENG 3734 RUN EXTRA CALIENTE TO BAKERSFIELD

Extra 3734 West will not leave Caliente until Extra 6236 East has arrived.  Except when otherwise provided, the above examples give the extras no authority to occupy the main track at the turning point of fulfillment of the order.

H

WORK EXTRAS

(1) ENG 3611 WORKS EXTRA 1145 AM UNTIL 301 PM BETWEEN CALIENTE AND ALLARD

Work extra, whether standing or moving, must protect itself against extra trains within work limits in both directions.

Time of regular trains must be cleared.

This may be modified by adding:

(a) NOT PROTECTING AGAINST EASTWARD TRAINS

When above is added the work extra will protect only against westward extra trains.

(b) NOT PROTECTING AGAINST EXTRA TRAINS

When above is added protection against extra trains is not required.

(c) NOT PROTECTING AGAINST EXTRA TRAINS UNTIL 240 PM

When above is added protection against extra trains is not required until time named, and extra trains must not enter work limits before that time.

- - - - - - - - - - - - - - -

When a work extra has been authorized by train order not to protect against extra trains, and later it is desired that after a certain time it clear the track for an extra, or to protect against a designated extra in either direction, the appropriate example of the following may be used:

(2) WORK EXTRA 2362 CLEARS EXTRA 5471 EAST BETWEEN ILMON AND CALIENTE AFTER 210 PM

(3) WORK EXTRA 2362 PROTECTS AGAINST EXTRA 5471 EAST BETWEEN ILMON AND CALIENTE AFTER 210 PM

To enable a work extra to follow an extra into work limits when it is not practicable to furnish the extra a copy of the work order, the following example may be used:

(4) WORK EXTRA 3711 MUST NOT PASS EXTRA 4101 WEST BETWEEN WALONG AND WOODFORD

- - - - - - - - - -- - - - - -

When necessary to authorize two or more work extras within the same limits or portions of the limits, the following example may be used, and responsibility for the thorough understanding as to the movements of two work extras, and protection that is to be provided will rest with conductors and engineers of the work extras named.

(5) ENG 5296 AND ATSF 2651 WORK EXTRA 1201 PM UNTIL 430 PM BETWEEN MAGUNDEN AND ARVIN NOT PROTECTING AGAINST EXTRA TRAINS EXCEPT PROTECTING AGAINST EACH OTHER

- - - - - - - - - - - - - - -

If it is desired to authorize two or more work extras within the same limits or portion of limits and permit one of the work extras to move without providing flag protection against other work extra or extra trains, the following example may be used:

(6) ENG ATSF 1683 AND ENG ATSF 195 WORK EXTRA 1130 AM UNTIL 430 PM BETWEEN MAGUNDEN AND ARVIN NOT PROTECTING AGAINST EXTRA TRAINS EXCEPT WORK EXTRA ATSF 1683 PROTECTS AGAINST WORK EXTRA ATSF 195

This permits work extra ATSF 195 to move within these limits without providing protection against work extra ATSF 1683 and restricts the movement of work extra ATSF 1683 except under flag protection against work extra ATSF 195.

- - - - - - - - - - - - - - -

To enable a work extra to work on time of a regular train, following example may be used:

(7) WORK EXTRA 5320 PROTECTS AGAINST NO 55 ENG 4193 BETWEEN CLIFF AND CALIENTE

Work extra may work on the time of regular train mentioned in order, and must protect against that train.

(8) WORK EXTRA 5324 HAS RIGHT OVER ALL TRAINS BETWEEN WOODFORD AND MARCEL 115 PM UNTIL 501 PM

This order gives work extra exclusive right between points designated, between times named, and other trains must not enter the limits unless permission is obtained from the conductor and engineer of work extra.

This order must not be modified or amplified, and must not be issued while an order, example (3) Form G, is in effect.

- - - - - - - - - - - - - - -

Work limits should be as short as practicable, to be changed as progress of work may require.

Work extras must give way to all trains as promptly as practicable.

Whenever extra trains are run into limits of a work extra they must be given a copy of the orders sent to the work extra.  Should work order instruct work extra not to protect against extra trains in one or both directions, extra trains must protect against work extra;  if order indicated that work extra is protecting itself against other trains, such trains must run expecting to find work extra protecting itself.

I

Relief of Protection by Flagman

(1) NO 447 ENG 4318 NOT PROTECTING AGAINST WESTWARD EXTRA TRAINS UNTIL 130 PM BETWEEN MOJAVE AND BAKERSFIELD

(2) EXTRA 5031 EAST NOT PROTECTED AGAINST EASTWARD EXTRA TRAINS UNTIL 130 PM BETWEEN EDISON AND CALIENTE

This form must not be used to relieve a train of protection by flagman to rear against a regular train, and must not be used when a work extra in territory named has been instructed not to protect against extra trains.

This form must not be combined with other form and must not be used in territory where, by special instructions in time table, trains moving with current of traffic may run extra or work extra without train order authority, as prescribed by Rule D-97.

J

Holding Order

(1) HOLD NO 52

(2) HOLD ALL TRAINS

When train has been so held it must not be permitted to proceed until order to hold is annulled, or an order is addressed to operator in following form:

(3) NO 52 MAY GO

These orders will be addressed to operator and acknowledged in the usual manner.

Form J will be used only when necessary to hold trains until other orders can be given, or in case of emergency.

K

Annulling a Schedule or Section

(1) NO 4 OF FEB 21 IS ANNULLED BAKERSFIELD TO MOJAVE

(2) SECOND 51 OF FEB 21 IS ANNULLED MOJAVE TO BAKERSFIELD

(3) NO 52 OF FEB 21 HAS ARRIVED WOODFORD AND IS ANNULLED WOODFORD TO MOJAVE

Schedule or section annulled becomes void between points named and cannot be restored.

When a section is annulled from an intermediate point, opposing inferior trains must not leave that point until it is ascertained that the schedule has been fulfilled to that point, or an order example (3) is received, or an order is received authorizing train to proceed.

L

Annulling an Order

Date named in the order must be the date on which schedule is due to leave its initial station on the subdivision.

(1) ORDER NO 10 IS ANNULLED

If an order which is to be annulled has been delivered to a train, the annulling order must be addressed to that train.  If it has not been delivered, annulling order may be addressed to operator, who must then destroy all copies of order annulled but his office copy, on which he must write the number of annulling order.

Operator or train addressed must have a copy of the order annulled.

When an order is annulled on other than date of issue, date as well as number of order annulled must be designated in the annulling order.

(2) THIS ORDER ANNULLED AT 210 PM

Above addition may be made to any order which is to be annulled at a predetermined time.

An order which has been annulled must not be reissued under its original number.

M

Annulling Part of an Order

(1) THAT PART OF ORDER NO 10 READING NO 23 ENG ATSF 51 MEET NO 58 ENG 4294 AT ROWEN IS ANNULLED

(2) THAT PART OF ORDER NO 21 READING EXTRA 5484 WEST WILL NOT LEAVE TEHACHAPI UNLESS EXTRA ATSF 170 EAST HAS ARRIVED IS ANNULLED

(3) THAT PART OF ORDER NO 12 READING NO 57 ENG 4165 WAIT AT WALONG UNTIL 420 PM IS ANNULLED

Form M must be used when a particular movement or portion of movement in an order is to be annulled, and does not affect other movements in the order.

P

Superseding an Order or Part of an Order

(1) NO 55 ENG 5333 MEET NO 58 ENG 6440 AT ROWEN INSTEAD OF WOODFORD

(2) NO 447 ENG 6190 PASS NO 55 ENG 4229 AT CLIFF INSTEAD OF BEALVILLE NO 55 TAKE SIDING

(3) EXTRA ATSF 153 WEST HAS RIGHT OVER FIRST 804 ENG 6345 TEHACHAPI  TO ALLARD INSTEAD OF BENA

(4) ORDER NO 14 SHOULD READ EXTRA ATSF 3854 WEST INSTEAD OF EXTRA ATSF 3936 WEST

(5) NO 52 HAS ENG 6390 INSTEAD OF ENG 4350

Train addressed must have a copy of order containing movement that is superseded.

Meeting point must not be changed by use of Form P train order more than once.  If further changes are necessary, previous order must be annulled.

This form must be used whenever limits in an order Form S-C, are shortened or lengthened.

An order which has been superseded must not be reissued under its original number.

When a train is directed by train order to take siding or to hold the main track for another train, such instructions apply only to point named in order, and do not apply to superseding order unless so specified.

Q

Issuance of New Timetable or Supplement

(1) TIMETABLE NO 73 IS EFFECTIVE 1201 AM JAN 1

(2) SUPPLEMENT NO 1 TO TIMETABLE NO 3 IS EFFECTIVE 1201 AM FEB 15

D-R

Providing for a Movement Against Current of Traffic

(1) NO 51 ENG 4352 HAS RIGHT OVER OPPOSING TRAINS ON EASTWARD TRACK BENA TO EDISON

The designated train must use the track specified between points named and has right over opposing trains on that track between those points.  Opposing trains must not leave point last named, nor any intermediate point where the order is received, until the designated train arrives.

All trains between points named moving with current of traffic in the same direction as designated train must, when practicable, be given a copy of the order, and they may then proceed on their own schedules, or authority.

- - - - - - - - - - - - - - -

This order may be modified as follows:

(2) AFTER NO 4 ENG ATSF 3745 ARRIVES AT BENA NO 51 ENG 4352 HAS RIGHT OVER OPPOSING TRAINS ON EASTWARD TRACK BENA TO EDISON

Train to be moved against current of traffic must not leave first-named point until arrival of the first-named train.

- - - - - - - - - - - - - - -

A train must not be moved against the current of traffic until track on which it is to run has been cleared of opposing trains, or until those trains have in their possession a copy of order authorizing movement.

Where trains moving with current of traffic are authorized to run extra or work extra without train-order authority, Form D-R must not be used until all trains and engines which may use either track, have received copy of the order.

V

Check of Trains

(1) REGULAR TRAINS DUE KERN JCT BEFORE 330 PM JUNE 4 HAVE ARRIVED AND LEFT EXCEPT THIRD 806

“Due” refers to schedule arriving or leaving time, and not to run or wait order time.

(2) NO 58 OF JUNE 4 HAS ARRIVED MOJAVE

(3) THIRD 4 OF JUNE 4 HAS LEFT KERN JCT WITH GREEN SIGNALS

A check of a section will indicate that all preceding sections of the same schedule have also arrived or left, and must state “with no signals” or “with green signals”, as the case may be.

When necessary to make an exception of a train or trains, such exception will be added.  Train addressed is not to be included in the exception.

Where not required, the work “arrived” or “left” will be omitted.

When this form is received train may register by ticket, provided it is not necessary to check the register against other schedules.

Y

Speed Limit Orders

(1) 1201 PM TO 501 PM APPROACH GANGS OR MACHINES BETWEEN MP 335.5 AND MP 336 BETWEEN CALIENTE AND ALLARD PREPARED TO STOP UNLESS PROPER PROCEED SIGNAL RECEIVED

To be used covering work where safety to the movement requires that trains approach prepared to stop, and are not to proceed until authorized by proceed signal given by flagman.

(2) SPEED LIMIT 10 MPH BETWEEN MP 338 AND MP 339.5 BETWEEN CALIENTE AND BEALVILLE

To be used where track condition required speed to be reduced below normal.

(3) SPEED LIMIT 15 MPH OVER BRIDGE MP 335.9 BETWEEN CALIENTE AND ALLARD

To be used where bridge conditions require speed to be reduced below normal.

Z

Signals Taken Out of Service or Restored

(1) ABS OUT OF SERVICE BETWEEN SIGNAL 333.2 AND SIGNAL 334.2 RULES GOVERNING MOVEMENT OUTSIDE OF BLOCK SYSTEM LIMITS WILL APPLY SPEED OF PASSENGER TRAINS MUST NOT EXCEED 59 MPH AND OTHER TRAINS 49 MPH

(2) CTC OR INTERLOCKING OUT OF SERVICE BETWEEN EASTWARD A SIGNAL EAST END ILMON AND WESTWARD A SIGNAL EAST END BEALVILLE RULES GOVERNING MOVEMENT OUTSIDE OF BLOCK SYSTEM LIMITS WILL APPLY SPEED OF PASSENGER TRAINS MUST NOT EXCEED 59 MPH ALL OTHER TRAINS 49 MPH

(3) ABS AGAIN IN SERVICE BETWEEN SIGNAL 333.2 AND SIGNAL 334.2

(4) CTC AGAIN IN SERVICE BETWEEN EASTWARD A SIGNAL EAST END ILMON AND WESTWARD A SIGNAL EAST END BEALVILLE

Examples (1) and (2) may be prefaced by words “EFFECTIVE ______M ______ 19 ___” when necessary.

Examples (1) and (2) only to be used when it becomes necessary to remove a section of Automatic Block System or Centralized Traffic Control from service, trains receiving these orders will be governed by Rule 304.

Examples (3) or (4) will be used when restoring Automatic Block System or Centralized Traffic Control to service.

This form must not be combined with other forms of orders.

BLOCK SIGNAL AND INTERLOCKING RULES

Rules Governing the Movement of Trains and Engines in the Same Direction by Block Signals

D-251.  On portions of the railroad on designated tracks as specified in the timetable, trains will run with reference to other trains in the same direction by block signals whose indications will supersede the superiority of trains.

D-253.  Train dispatcher must be informed in advance of any known condition that will delay the train or prevent it from making usual speed.

D-254.  Except as affected by Rule D-251 all block signal rules and other rules remain in force.

Fixed Signals

Automatic block signals will have a number plate attached to their mast.

Interlocking signals will not bear number plates, but may have a plate bearing the letters “SA”.

Absolute signals will not bear number plates, but may have plates bearing “A” or “SA”.

Aspects as illustrated or referred to in these rules are shown by the position of the semaphore arms or color of lights, or both, as seen from an approaching train.  Other combinations may be used.

RULE 281.

Indication:  PROCEED

Name and Aspect:  BLOCK SIGNAL GREEN

RULE 282.

Indication:  PROCEED EXCEPT ON DIVERGING ROUTE

Name and Aspect:  BLOCK SIGNAL GREEN

RULE 283.

Indication:  PROCEED ON DIVERGING ROUTE

Name and Aspect:  BLOCK SIGNAL GREEN FOR DIVERGING ROUTE

RULE 284.

Indication:  REDUCE TO MEDIUM SPEED AND PROCEED.  NEXT SIGNAL INDICATES “PROCEED ON DIVERGING                  	        ROUTE”

Name and Aspect:  BLOCK SIGNAL YELLOW OVER GREEN

RULE 285.

Indication:  PROCEED NOT EXCEEDING MEDIUM SPEED, PREPARED TO STOP SHORT OF NEXT HOME 	   	       	       SIGNAL

Name and Aspect:  APPROACH SIGNAL YELLOW

Trains exceeding medium speed must reduce to medium speed before engine reaches the signal if advance view of the signal permits.  After entering the block speed may be resumed provided next signal can be seen displaying proceed indication (green aspect).

RULE 287.

Indication:  REDUCE TO RESTRICTED SPEED AND PROCEED

Name and Aspect:  GRADE SIGNAL RED

RULE 288.

Indication:  PROCEED ON DIVERGING ROUTE AT RESTRICTED SPEED

Name and Aspect:  HOME SIGNAL YELLOW FOR DIVERGING ROUTE

RULE 290.

Indication:  STOP

Name and Aspect:  HOME SIGNAL RED

RULE 293.

Indication:  STOP.  INSPECT TRAIN, TRACK OR STRUCTURE AS PROVIDED IN SPECIAL INSTRUCTIONS, AND PROCEED THROUGH 	     REMAINDER OF THE BLOCK IN ACCORDANCE WITH PROVISIONS OF RULE 507, 663, 744, OR 776, AS THE CASE MAY BE

Name and Aspect:  HOME SIGNAL RED, WITH TRIANGULAR PLATE

301.  A signal improperly displayed, or absence of a light where one should be, must be regarded as the most restrictive indication that can be given by that signal, and must be reported to train dispatcher.

303.  Interlocking or absolute signals may govern movements into ABS adjoining interlocking or CTC limits.  When so arranged they will be designated “semi-automatic” and distinguished by a plate bearing the letters “SA”.  Trains stopped by such signals must observe interlocking or CTC rules within interlocking or CTC limits, and ABS rules within automatic portion of the block beyond interlocking or CTC limits.

304.  When automatic block, absolute or interlocking signals are to be taken out of service for an extended period of time, train order, Form Z example (1) or (2), as the case may be, and timetable bulletin must be issued.  Lights in signals will be extinguished or covered and arms removed and lights extinguished in semaphore signals, or lights covered in light  signals.

When signals are to be again restored to service, train order, Form Z example (3) or (4), as the case may be, and timetable bulletin must be issued.

305.  After passing a signal indicating “proceed”, indication of the next signal may change to “stop”.  Enginemen and trainmen must be on the alert to observe it.

A train, if delayed in the block, must proceed with caution to the next signal.  Maximum authorized speed may be resumed after head end passes signal displaying proceed indication.

306.  A home signal with triangular plate bearing letter “P” is also actuated by some special protective device, and when signal displays stop indication, Rule 507, 663, 744 or 776, as the case may be, will govern.  In addition, careful examination must be made of train, track or structure for which protection is provided to assure that it is safe for the passage of trains.  Number or location of such signals will be shown in timetable, with description of the special protection provided.

When a signal with triangular plate protecting a spring switch displays stop indication, except when the switch is lined by hand for the movement, member of crew must open and close spring switch by hand, removing any obstruction.

AUTOMATIC BLOCK SIGNAL SYSTEM

505.  Automatic block signals govern the use of blocks, but unless otherwise provided, do not supersede the authority of trains; nor dispense with the use or observance of other signals whenever and wherever they may be required.

D-506.  Unless otherwise provided, block signals for a track apply only to trains moving with the current of traffic on that track.

507.  When an automatic block signal displays stop indication, train, after stopping, may proceed at restricted speed.  Under the following conditions train may proceed at restricted speed without stopping:

(a) To enter a siding where switch is not more than one thousand (1000) feet beyond the signal and it can be seen that the siding is clear, or 	that an opposing or preceding train has stopped, or is prepared to stop clear of the route to be used.

(b) To enter a yard track when switch is lined for the receiving track and route is clear to the fouling point of switch.

(c) To continue on main track when meeting or passing a train.

(d) To continue within limit of a grade signal.

(e) To permit an engine with, or without cars, to couple to its train.

When proceeding under provisions of Rule 507, restricted speed must not be exceeded until rear of train has passed out of the block or has passed a signal displaying green aspect.

513.  Before a train or engine fouls a main track and before main track switch is thrown it must be known by observance of signal, or by view of entire length of the block that there is no train or engine, either within or closely approaching the block to be occupied; otherwise protection by a flagman must be provided.  Engineer or conductor, as the case may be, at the fouling point must know that such protection by flagman has been provided, except in yards where switchman’s proceed signal is required for trains to enter main track, such signal is also an indication that such protection has been provided.  Switchman giving signal must know that such protection has been provided.

514. A train or engine entering a block between signals must be properly protected as required by the rules, and must proceed with caution to the next signal.

515.  A train having passed out of a block must not back into that block except under protection by flagman.

Reverse movement within a block must not be made except under protection by a flagman.

When making a reverse movement on main track after moving out of a siding or other track, train will, unless movement is completed beyond the governing signal, proceed only as prescribed by the applicable portion of Rule 507.

517.  Insulated joints are placed near the fouling point.  Engines or cars must not be left between insulated joints or fouling point and switch.

SPRING SWITCHES

535.  A spring switch will be distinguished by a target bearing letters “SS”.  After trailing through a spring switch reverse movement must not be made until it is known that both points have moved to the proper position.  When stop is made before the entire movement is completed, reverse movement must not be made, nor slack taken, until switch has been lined by hand.

Maximum speed for trailing movements when spring is to be actuated, and maximum speed for facing movements with switch points in normal position, as indicated in speed restrictions tables must not be exceeded.

536.  Spring switches must not be trailed through unless switch target displays the letters “SS” in normal position, or switch has been lined for the movement.

538.  Location and normal position of spring switches on main tracks will be listed in timetable.

INTERLOCKING

605.  Interlocking signals govern the use of routes of an interlocking, and as to movements within interlocking limits, their indications supersede the superiority of trains, but do not dispense with the use or observance of other signals whenever and wherever they may be required.

Rule D-152 will not apply within interlocking limits.

Movements governed by diverging route or dwarf signals must be made with caution and position of switches observed as such signals may govern movements for various routes.

606.  Interlocking limits on main tracks will be designated in timetable, except at railroad crossings, junctions and drawbridges where limits are clearly defined by interlocking home signals.  Where necessary, limits on other than main tracks will be indicated by signs.

607.  Automatic signals located within interlocking limits will be observed in accordance with Rule 507.

608.  When power-operated switches are dual control type, Rule 771 and Rule 772 will also apply.

611.  Unless otherwise provided, signals must be kept in position displaying the most restrictive indication, except when displayed for an immediate movement.

615.  When necessary to change any route for which the signals have been cleared for an approaching train, switches, moveable point frogs or derails must not be changed, nor signals cleared for conflicting route, until the train for which the signals were first cleared has stopped.

621.  Operators must observe, as far as practicable, whether indication of signals and the position of derails and switches correspond with position of levers.

626.  Operators must observe, as far as practicable, each passing train, and note whether it is complete and in order.

627.  When an interlocking signal displays stop indication because of track occupancy by train or engine, operator may authorize another train or engine to enter block if necessary, provided he has assured himself that the train or engine is fully acquainted with the intended move.

629.  Before granting permission in accordance with Rule 663 for trains or engines to pass an inoperative signal, red tag or wedge must be applied to signal lever, and it must not be removed until it is known that train or engine has completed the authorized move.

633.  If a train overruns a signal displaying stop indication or fails to obey a hand signal, the fact must be reported to superintendent.

636.  A train-order signal with the limits of an interlocking must not be cleared for an approaching train until the preceding interlocking signals governing the movement of the train have been cleared.

661.  If a signal indication permitting a train to proceed has been accepted, and is changed to display stop indication before it is reached, stop must be made at once.  Such occurrences must be reported to superintendent by engineer.

If for any reason proceed indication of an interlocking signal cannot be acted upon at once, operator must immediately be notified.

662.  Interlocking limits must not be entered or fouled on any track unless properly authorized by signal indication, or by permission of operator.

663.  When an interlocking signal displays stop indication, train or engine, after stopping, may proceed through the interlocking limits of such signal at restricted speed upon receiving authority from operator, and after a member of the crew has made careful examination of switches and derails.

669.  Trains stopped by operator in making a movement through an interlocking, must not move in either direction until they have received proper authority from operator.

670.  A reverse movement within interlocking limits, or a forward movement after making a reverse movement, must not be made without the proper interlocking signal indication or permission from operator.

LETTER-TYPE INDICATORS

705.  Letter-type indicators may display indications by illuminated letters, and when so displayed require movement as shown in special instructions in timetable.  They supersede the superiority of trains to the extent shown in such special instructions.  Restrictions that may be imposed by automatic block signals or other signals must be complied with.

Fig. 1			Fig. 2

Following letters may be used in these indicators:

H

M

S

SS

NS

W

T

Other letters or combination of letters, or words may be used.

706.  At point where authority granted by letter-type indicator terminates, or when no letter is illuminated in an indicator, timetable and train-order requirements will govern.

707.  At stations where indicator is actuated by operator, it must not be illuminated without authority from train dispatcher, except those governing movements for which yardmaster’s authority must be obtained.

ABSOLUTE-PERMISSIVE BLOCK

740.  On portions of the railroad within limits specified in the timetable and designated as absolute-permissive block, trains will be governed by block signals whose indications will supersede the superiority of trains for both opposing and following movements on the same track.

741. Absolute signals govern use of routes within A-PB limits, and confer authority to the limit of A-PB; but do not dispense with the use of observance of other signals whenever and wherever they may be required.

744.  When an absolute signal displays stop indication train must stop.  After waiting four (4) minutes and no train or engine is seen or heard approaching, train may proceed with caution to next signal displaying proceed indication or when he has reached opposite end of A-PB.

CENTRALIZED TRAFFIC CONTROL

NOTE:  Where, in these rules, the term “train dispatcher” is used it has reference to the train dispatcher or operator or other employee acting upon authority of train dispatcher.

760.  On portions of the railroad within block system limits specified in timetable and designated as centralized traffic control, trains will be governed by block signals whose indications supersede the superiority of trains, unless otherwise provided.

761.  Absolute signals govern the use of routes within CTC limits; and confer authority to next absolute signal, except that the last absolute signal confers such authority only to CTC limits.  They do not dispense with the use or observance of other signals whenever and wherever they may be required.

762.  Within CTC limits, Rule 93 will not apply.

764.  Initial and terminal points of CTC will not affect initial and terminal stations of schedules, nor of extra train order.  Rule 96 must be observed in creating sections.

765.  When necessary to perform switching and track is unoccupied, train dispatcher must designate the work limits and clock time limit that must not be exceeded.

Trains or engines granted work limits and clock time limit may occupy main track and move in either direction within such limits with caution, and without protection by flagman, passing automatic block signals displaying stop indication without stopping, but all movements must be made with caution.

Unless granted an extension of time, a train or engine must be clear and reported clear of the work limits before clock time limit given.  If impossible to clear the limits by the time specified, protection by flagman, in both directions, must be provided as per Rule 99.

When train or engine crew requests work and clock time limits or reports clear of work limits, employee will state name, occupation, location, and train or engine number.  Work and time limits must be reported to train dispatcher, and if correct, response “OK” will be given.

766.  Levers controlling a dual control switch must not be actuated while an engine, car or any portion of a train is standing on, or moving over the switch.

769.  Main track or a controlled siding must not be fouled unless authorized by absolute signal indication or by permission from train dispatcher.  Certain main track hand-operated switches are equipped with electric switch locks, and train dispatcher’s permission must be obtained before door on high-type switch lock is opened, or switch lock removed from keeper of low-type switch lock.

Before granting authority to enter main track at a hand-operated switch, train dispatcher must protect movement by placing absolute signal in stop position.

770.  When two or more trains or engines are to be granted work and clock time limits within the same or overlapping territory train dispatcher must inform conductor of each train or engine of this fact and conductors must arrange for necessary protection.

Train dispatchers must not authorize train or engine to pass an absolute signal displaying stop indication for a following move until he has assured himself that conductor of any train already within the block is fully acquainted with the intended move so that proper protection will be provided.

771.  Train dispatcher must not operate a dual control switch for switching moves requiring more than one reverse and one normal movement of the switch.

When switching is to be done over dual control switch, work and clock time limits must be obtained from train dispatcher.

Train or engine granted work and clock time limits may consider indication of such signals suspended and make movements over the switch on hand signals from trainmen.  Engineer will be governed by hand signals while making movements over the switch.

772.  When necessary to hand operate a dual control switch permission from train dispatcher must be first be obtained, then be governed by instructions posted at switch.

774.  To enter main track at hand-operated switches where absolute signal is provided to govern movement, permission for movement must first be obtained from the train dispatcher, then line switch and be governed by absolute signals indication.

775.  If a change in direction of movement is to be made while any portion of train is occupying, or which will cause any portion of train to occupy, the short section of track between opposing absolute signals at a dual control switch while selector lever is in motor position, train dispatcher’s permission must first be obtained, and it must be known that switch points for either facing or trailing movement are in proper position for the movement.

Reverse movement on controlled siding must not be made without first obtaining permission from train dispatcher.

776.  When an absolute signal displays stop indication, train or engine must stop. Unless stop indication is known to be caused by approach of a train, train dispatcher must be consulted promptly by member of crew and the following will govern:

(a)  If signal cannot be cleared and there is no opposing or preceding train causing signal to display stop indication and light on control machine is illuminated (indicating that dual control switch is locked), and provided time-release feature is not operating and movement does not require position of switch to be changed, train dispatcher, after placing opposing absolute signals in stop position, may authorize train to proceed on main track or siding at restricted speed to next absolute signal and trains may pass automatic block signals displaying stop indication without stopping.  Member of crew must examine switch to see that points are in proper position for movement.

(b)  If desired movement requires that position of switch be changed or if light on control machine is not illuminated (which would indicate dual control switch is not locked), train dispatcher must not authorize movement until dual control switch has been opened and closed by hand.  If signal cannot then be cleared, member of crew must notify 	train dispatcher when selector lever has been returned to motor position.

(c) Trains authorized by  train dispatcher to pass absolute signal displaying stop indication, and  to proceed at 	restricted speed under the provisions of this rule, may resume maximum authorized speed after rear of train passes next block signal displaying proceed indication.

777.  If for any reason proceed indication of an absolute signal cannot be acted upon at once, train dispatcher must immediately be notified.

781.  Instructions relating to track conditions will be issued by train order before train enters CTC when practicable, but when not practicable, may be issued orally to engineer within CTC for such conditions as apply within CTC limits.  Conductor of a train, or engineer if train is a light engine, entering CTC at an intermediate point, must ascertain from train dispatcher what instructions are in effect as to track conditions on that portion of CTC over which movement is to be made.

GENERAL REGULATIONS

801.  Employees will not be retained in the service who are careless of the safety of themselves or others, indifferent to duty, insubordinate, dishonest, immoral, quarrelsome or otherwise vicious, or who conduct themselves in a manner which would subject the railroad to criticism.  Courteous deportment is required of all employees in their dealings with the public, their subordinates and each other.  Boisterous, profane or vulgar language is forbidden.

804.  Any act of hostility, misconduct or willful disregard or negligence affecting the interest of the Company will not be condoned and must be reported.  Withholding such information will be considered proof of negligence or indifference, and treated accordingly.

809.  The railroad’s communication system is for handling railroad business, but may be used for messages relating to personal affairs of employees in cases of illness or accident.  Commercial telephones on railroad property, except pay phones, are not to be used without permission from proper authority and long distance or message unit calls are not to be made unless specifically authorized.  The railroad’s office equipment and machines must not be used other than for railroad business.

810.  Employees must report for duty at the prescribed time and place, remain at their post of duty, and devote themselves exclusively to their duties during their tour of duty.  They must not absent themselves from their employment without proper authority.

811.  Employees whose duties connect them with the movement of trains or engines must not absent themselves from their places, substitute others, or exchange duties without proper authority.  Trains must not be delayed for trainmen and enginemen to eat, without permission of chief train dispatcher.

820.  In case of damage to trains, engines, cars or structures involving security of freight or other company property, it is the duty of all employees to see that arrangements are promptly made for its protection against loss through pilferage or other causes.

824.  On grades before engine is cut off, a sufficient number of hand brakes must be set to hold the train.

825.  Cars must not be left on sidings without permission of train dispatcher.

Cars must be kept clear of any street or public crossing, and at least one hundred (100) feet from crossing when practicable.

827.  Speed of freight trains must not exceed eight (8) miles per hour when starting from initial stations and intermediate stops, for length of train.  Trainmen and enginemen must be in position to observe their train while running.

829.  Employees must observe passing trains closely and if hot bearings, brakes sticking, wheel sliding, dragging equipment, insecure lading or any other dangerous condition is detected, they must give stop signals to trainmen and enginemen on passing train.  If nothing irregular is noted, they will give proceed signal as rear of train passes, as an indication they have observed train and noted nothing dangerous.

830.  Trains and engines when standing or switching must not block traffic on street or public road crossings longer than necessary.

Trains must not stand on railroad crossings at grade or within the limits of an interlocking, when practicable to avoid it.

831.  Pile drivers, locomotive cranes, and other work equipment having butt couplers (no draft gear) and scale test cars must be handled near rear of train ahead of caboose and behind any helper engine.  When practicable all other work equipment, including steel underframe outfit cars, power shovels, derricks, spreaders, pile drivers and locomotive cranes should be similarly handled in the train.

834.  Open top cars loaded with rail, pipe, logs, lumber, structural steel, poles or mounted wheels, when lading projects above the sides or end walls of cars, must not be placed in trains next to engines, cabooses, occupied cars, nor cars placarded “Explosives”, “Dangerous”, “Poison Gas” or “Flammable Poison Gas”.  This does not apply to trains consisting entirely of logs.  Federal regulations must be observed in handling placarded cars in proper location in train.

836.  Outside of yard limits cars must not be shoved ahead of engine between stations when it can be avoided.  When necessary to so handle, speed must not exceed 20 MPH.

837.  Switching must be carefully done, and trains and engines must be carefully handled, to avoid shocks from abrupt starting or stopping; from impact in making coupling, and to prevent personal injuries, and damage to equipment or contents.

Before fouling any track it must be known that engines or cars on adjacent tracks are clear.  Cars must not be shoved to foul leads or adjacent tracks until it is known that it is safe to do so.  Cars and engines must not be left foul of an adjacent track if possible to avoid it.

Movements into spur tracks must be controlled to prevent damage at end of spur.

842.  Yardmasters are responsible for conditions within yards.  Trains and engines are under control of the yardmaster, and all employees in train, engine and yard service will be subject to his direction as to movements within yard limits.  Road crews of trains will be responsible for their respective trains and engines until yardmaster or his representative takes charge.

871.  Engineers and conductors must assure themselves that subordinate employees on engines and trains are competent and instruct them if necessary in the performance of their duties.  Incompetence and disobedience must be reported.

874.  Enginemen and trainmen on engine must be alert in all matters pertaining to safety and while running must keep a vigilant lookout, carefully note all signals, observe position of switches and derails affecting their movement, and watch for obstructions and defects in track.  They must frequently look back for signals and indications of defects in train, especially while rounding curves and approaching or leaving stations.

883.  Trains, engines or cars must not be left on main track without protection, nor left on any track blocking movement on adjacent tracks.

896.  The loading of lumber, timber, scrap metal and other freight on open top cars must be in conformity with Rules Governing the Loading of Commodities on Open Top Cars as published by the Association of American Railroads, printed copies of which will be furnished for use of shippers upon application to the Superintendent.

915.  Unless otherwise provided, operators are subordinate to chief train dispatcher and will give train orders preferred attention.  They must not absent themselves or close office without permission from the train dispatcher, except when means of communication fail.

RAILROAD RADIO RULES

GENERAL

950.  Following rules and requirements cover the use of railroad radio systems and govern employees using such system.

OPERATING RULES

952.  No employee shall knowingly transmit any false distress communication, any unnecessary, irrelevant or unidentified communication, nor utter obscene, indecent or profane language via the radio.

954.  Before transmitting, any employee operating a radio transmitting set shall listen a sufficient interval to be sure that the circuit is not already in use, particularly for distress traffic.

When a train order is being transmitted to a train by radio, employees not being addressed shall not use the radio, except in case of emergency, until the train order has been completed.

958.  Employees, except in yard conditions, shall identify the radio station from which they are calling by prefacing their call with the railroad name, for example:  “SP Caboose, Train Second 802 calling SP Engine Second 802, over” and to answer a call, announce for example: “This is SP Caboose, Train Second 802, over”.

961.  The use of radio is restricted to transmission of information and instructions essential to railroad operation.

962.  Radio communication system may be used in lieu of hand, flag or lamp signals prescribed by Rule 12 when such signals cannot be passed to engineer.  When movement is being controlled by use of radio, continuous voice transmission must be maintained, using train or engine identification in conjunction with the signal; and movement must be made with caution.  Failure of engineer to receive continuous voice transmission must be construed as a stop signal.

963.  Radio communication, if distinct, may be used the same as any other means of communication to effectuate the following:

(a) Operator communicating direct with engineer, after assured train is stopped, may authorize train to pass an interlocking signal displaying 	stop indication, as prescribed by Rule 663 (b).

(b) Operator communicating direct with engineer may authorize train to make reverse or forward movement within interlocking limits when 	no interlocking signal is provided to authorize movement, as prescribed by Rule 670.

(c) Train dispatcher communicating direct with engineer, after assured train is stopped, may authorize train to pass an absolute signal 	displaying stop indication within CTC limits as prescribed by Rule 776.

(d) Train dispatcher communicating direct with conductor may designate work limits and clock time limit as 	prescribed by Rule 765.

965.  Radio communication, when distinct, may be used as herein provided:

(a) Train dispatcher may transmit train order to train-order operator as prescribed by Rule 206-A.

(b) Train-order operator may relay train order as prescribed by Rule 206-C.

966.  When radio communication system is used, employee must plainly identify himself by giving his name, occupation, location and when applicable, schedule or engine number.  Train dispatchers and operators must not transmit information to a train until employee requesting information has properly identified himself.